FIELDPIX

# Pre-drywall inspection checklist

fieldpix.org/guides/pre-drywall-inspection-checklist

Job / address: ______________________________

Walked by: ______________________________

Date: ______________________________

With: ______________________________

## Before you start

| Check | OK / Fix / N/A | Notes | Photo # |
| --- | --- | --- | --- |
| Plans, electrical layout and selections on hand | ☐ OK ☐ Fix ☐ N/A |  |  |
| Rough inspections passed (framing, electrical, plumbing, mechanical) | ☐ OK ☐ Fix ☐ N/A |  |  |
| Insulation not yet installed | ☐ OK ☐ Fix ☐ N/A |  |  |

## Framing

| Check | OK / Fix / N/A | Notes | Photo # |
| --- | --- | --- | --- |
| Studs straight and plumb, nothing split or cracked | ☐ OK ☐ Fix ☐ N/A |  |  |
| Headers over every window and door | ☐ OK ☐ Fix ☐ N/A |  |  |
| Window and door openings sized for the units ordered | ☐ OK ☐ Fix ☐ N/A |  |  |
| Notches and holes within limits, or repaired | ☐ OK ☐ Fix ☐ N/A |  |  |
| Hangers, straps and hold-downs fully nailed | ☐ OK ☐ Fix ☐ N/A |  |  |
| Blocking for cabinets, grab bars, TV, handrails and towel bars | ☐ OK ☐ Fix ☐ N/A |  |  |
| Fireblocking where required | ☐ OK ☐ Fix ☐ N/A |  |  |
| No water stains on sheathing or roof deck | ☐ OK ☐ Fix ☐ N/A |  |  |

## Electrical

| Check | OK / Fix / N/A | Notes | Photo # |
| --- | --- | --- | --- |
| Every box where the plans put it, at the planned height | ☐ OK ☐ Fix ☐ N/A |  |  |
| Boxes set out for the finished wall (thicker for tile) | ☐ OK ☐ Fix ☐ N/A |  |  |
| Cables stapled, not crushed, nail plates where needed | ☐ OK ☐ Fix ☐ N/A |  |  |
| Dedicated circuits for appliances | ☐ OK ☐ Fix ☐ N/A |  |  |
| Outlets for counters, island and exterior walls | ☐ OK ☐ Fix ☐ N/A |  |  |
| Low voltage: data, TV, doorbell, cameras, speakers | ☐ OK ☐ Fix ☐ N/A |  |  |
| Smoke and CO detector locations | ☐ OK ☐ Fix ☐ N/A |  |  |

## Plumbing and gas

| Check | OK / Fix / N/A | Notes | Photo # |
| --- | --- | --- | --- |
| Supply and drains placed for the fixtures ordered | ☐ OK ☐ Fix ☐ N/A |  |  |
| Toilet distance from the wall | ☐ OK ☐ Fix ☐ N/A |  |  |
| Shower valve and head heights | ☐ OK ☐ Fix ☐ N/A |  |  |
| Tub and shower set, level and protected | ☐ OK ☐ Fix ☐ N/A |  |  |
| Pressure test passed, no drips or stains | ☐ OK ☐ Fix ☐ N/A |  |  |
| Nail plates where pipes are close to the stud face | ☐ OK ☐ Fix ☐ N/A |  |  |
| Gas lines capped and tested | ☐ OK ☐ Fix ☐ N/A |  |  |
| Washer box, ice maker line and hose bibs | ☐ OK ☐ Fix ☐ N/A |  |  |

## Heating, cooling and ventilation

| Check | OK / Fix / N/A | Notes | Photo # |
| --- | --- | --- | --- |
| Ducts connected, sealed and supported | ☐ OK ☐ Fix ☐ N/A |  |  |
| No crushed or kinked flex duct | ☐ OK ☐ Fix ☐ N/A |  |  |
| Registers and returns where planned | ☐ OK ☐ Fix ☐ N/A |  |  |
| Bath fans, range hood and dryer vented outside | ☐ OK ☐ Fix ☐ N/A |  |  |

## Photos

| Check | OK / Fix / N/A | Notes | Photo # |
| --- | --- | --- | --- |
| Every wall of every room, floor to ceiling | ☐ OK ☐ Fix ☐ N/A |  |  |
| Tape measure in the frame at a known height | ☐ OK ☐ Fix ☐ N/A |  |  |
| Corners and ceilings | ☐ OK ☐ Fix ☐ N/A |  |  |
| Photos labeled by room | ☐ OK ☐ Fix ☐ N/A |  |  |

## Sign-off

| Check | OK / Fix / N/A | Notes | Photo # |
| --- | --- | --- | --- |
| Fix items on a punch list with who and when | ☐ OK ☐ Fix ☐ N/A |  |  |
| Fix items re-checked | ☐ OK ☐ Fix ☐ N/A |  |  |
| Cleared for insulation and drywall | ☐ OK ☐ Fix ☐ N/A |  |  |

Free template from fieldpix.org. FieldPix can write this for you from a walkthrough on site.